# RTOS Fundamentals

Task Implementation:

- Task creation:

BaseType_t xTaskCreate(

TaskFunction_t pvTaskCode,

const char * const pcName,

const configSTACK_DEPTH_TYPE uxStackDepth,

void *pvParameters,

UBaseType_t uxPriority,

TaskHandle_t *pxCreatedTask);

- TaskFunction_t pvTaskCode – Pointer to the task entry function

- const char * const pcName – descriptive name of the task, The maximum length of a task's name is defined by configMAX_TASK_NAME_LEN in FreeRTOSConfig.h.

- const configSTACK_DEPTH_TYPE uxStackDepth – task stack size, the stack will be allocated in heap managed by freertos. The number of words (not bytes!) to allocate for use as the task's stack.

- void *pvParameters - A value that is passed as the paramater to the created task.

- UBaseType_t uxPriority – priority of the task, ranges from 0 - (configMAX_PRIORITIES - 1)

- TaskHandle_t *pxCreatedTask - Used to pass a handle to the created task out of the xTaskCreate() function. pxCreatedTask is optional and can be set to NULL.

- If the task was created successfully then pdPASS is returned, otherwise errCOULD_NOT_ALLOCATE_REQUIRED_MEMORY is returned.

- Example:

/* Create the task, storing the handle. */

xReturned = xTaskCreate(

vTaskCode, /* Function that implements the task. */

"NAME", /* Text name for the task. */

STACK_SIZE, /* Stack size in words, not bytes. */

( void * ) 1, /* Parameter passed into the task. */

tskIDLE_PRIORITY,/* Priority at which the task is created. */

&xHandle

); /* Used to pass out the created task's handle. */

Task Priority:

- Priority of task ranges from 0 to (configMAX_PRIORITIES - 1)

- Lower the priority lesser the urgency

- Helps the scheduler to which task run first

- Too many task priority lead to RAM over consumption(more context switching concurrently)

Scheduling:

- Pre-emptive scheduling

- Cooperative scheduling

- Pre-emptive scheduling – priority based & round robin

  - Round robin scheduling:

    - Cyclic manner

    - There will be ready task queue list

    - Schedule without priority

  - Priority based:

    - Task scheduled to run on the CPU based on their priority

    - Higher priority task will run the CPU for ever unless it get deleted/blocked/suspended or leaves voluntarily to other.

- Cooperative scheduling:

  - Task will run in the cooperative way

  - No automatic preemtion

- Scheduler code = Freertos generic code(in task.c) + arch specific code(port.c)

- In Scheduler, there will be SVC and pendsv handlers

- SVc handler will execute the first task, pendsv handler helps to switch between the task

Context switching:

- It is the process of switching out out of one task and switching in of another task on the CPu to execute

- In RTOS, Context Switching is taken care by scheduler

- It is taken care by pendsv handler.

- There will be priority base context switching

- Using taskYIELD() macro we can do the context switch manually

- State of the task -> Stack contents + Contents of the processor core registers

- When task execute on the CPU it uses

  - Processor core registers

  - Utilizes its own dedicated stack for push and pop operations

R0

R1

R2

R3

R4

R5

R6

R7

R8

R9

R10

R11

R12

R13

SP

LR

PC

PSR

PRIMASK

FAULTMASK

BASEPRI

CONTROL

R0 – R12 -> General Purpose Registers

SP – Stack pointer -> PSP – Process Stack Pointer -> MSP - Main Stack Pointer

- Task uses PSP to track task stack

- MSP for kernel

- All these registers need to be preserved before task switched out.

RAM_END									RAM_START

| Heap |  |  | Kernel Stack |  | Global Space |
| --- | --- | --- | --- | --- | --- |
| TCB-A | TCB-B | TCB-C |  |  |  |

- When Context switch happens (TASK switch in)

  - Processor Core registers will be saved on to the TCB automatically by the processor Systic interrupt entry sequence

  - Systick timer will pend the PendSV handler and Pendsv handler runs

  - Processor core registers(R4-R11, R14) saved manually to TCB

  - TCB has member element pxtopofstack

  - Save the new top of stack (psp) in to first member of TCB

  - Select next task to execute will be taken care by vTaskSwitchContext() which includes taskSELECT_HIGHEST_PRIORITY_TASK ( select new task to run and it will be optimised asm code)

- TASK SWICTH out

  - Load address of the Current TCB

  - Get the value of top of stack

  - Pop all the core registers (R4-R11,R14)

  - Exit the pendsv handler

  - Ehen pendsv unstacking will be happens, pop the core registers(R1-R3, PSR, PC,LSR)

IPC

- Queue: used to communicate between tasks

  - FIFO basis

  - There will be head and tail in the queue

  - Enqueue – add element to queue

  - Dequeue – retrieve element from queue

Task Synchronization and mutual exclusion

Kernal objects we ca used for synchronization:

- Events

- Semaphores (counting and binary)

- Queues and Message Queues

- Pipes

- Mailboxes

- Signals(UNIX like signals)

- Mutex

Semaphores:

- When semaphore is created, there will be SCB( Semaphore Control Block) contains Semaphore ID, values (binary or count) , task waiting list

- Semaphore will helps to block the critical section from executing it when another task already using it.

- If task acquires a semaphore then it can only execute the critical section

- If a task fails to get the semaphore, then the task will be blocked and kernel will put the task in to waiting list. Task will be changed from running to blocked state

- Binary semaphore, it has only semaphore value 0 or 1

- If semaphore value is 1, semaphore is available

- If value is 0, semaphore is not available

- Binary semaphore can be used for mutual exclusion, that is to guard the critical section

- Counting semaphore: the key value will be any number

- When the semaphore value reaches 0, then there will be no semaphore available, when a task tries to acquire the semaphore it will be block and sent to blocked list.

- If when a task releases the semaphore by decrementing the semaphore value. During this time the blocked task will be get the semaphore

- If more than one task available in the task waiting list, the higher priority task will uses the semaphore.

- If all the task in the waiting list has same priority, then first come first serve

Mutual exclusion

- Mainly used to protect the shared resources between the task

- We can use binary semaphore to implement mutex

- Also using mutes APIs

Common Mutex Problems:

- Deadlock: Two or more threads wait for each other to release resources (locks) they hold, resulting in a permanent standstill.

  - Example: Thread A locks Lock1 and waits for Lock2, while Thread B locks Lock2 and waits for Lock1.

- Starvation/Priority Inversion: A thread holding a mutex gets preempted or sleeps, preventing other threads (even high-priority ones) from acquiring the lock and making progress, causing them to wait indefinitely.

- Performance Overhead: Acquiring and releasing locks involves kernel operations (context switches), which can be slower than the actual protected work if the critical section is very short, reducing performance gains from threading.

- Thundering Herd: Many threads wake up simultaneously when a lock is released but only one gets it, with the rest immediately going back to sleep, wasting CPU cycles.

- Incorrect Handling:

  - Forgetting to unlock: Leaves the mutex locked forever, blocking all other threads.

  - Locking twice: A thread trying to lock a mutex it already holds causes a self-deadlock.

Solutions & Best Practices:

- Lock Ordering: Always acquire multiple locks in the same consistent order across all threads to prevent deadlocks.

- RAII (C++): Use lock guards (e.g., std::lock_guard, std::unique_lock) to automatically unlock mutexes when they go out of scope, even if exceptions occur.

- Minimize Critical Sections: Keep the code protected by the mutex as short as possible to reduce contention and overhead.

- Use Appropriate Tools: Consider alternatives like semaphores, condition variables, or lock-free data structures if mutexes aren't suitable for very short operations or high contention.